Lab 6: Build a Small Network in Packet Tracer — Answers

Lesson 6: Basic Networking and Segmentation

# 1. Objective

Apply networking concepts in an isolated Packet Tracer environment by creating a small network, assigning IP addresses, testing connectivity, documenting the topology, and proposing segmentation for staff and students.

# 2. Devices and Topology

One router: R1

One switch: S1

Four PCs: PC-Staff1, PC-Staff2, PC-Student1, PC-Student2

Optional server: Server-1

R1 (Router) ─── S1 (Switch) ─── PC-Staff1
                         ├── PC-Staff2
                         ├── PC-Student1
                         ├── PC-Student2
                         └── Server-1 (optional)

# 3. Configuration Table

| Device | Interface | IP Address | Subnet Mask | Gateway | Purpose |
| --- | --- | --- | --- | --- | --- |
| R1 | G0/0 | 192.168.10.1 | 255.255.255.0 | N/A | LAN gateway |
| S1 | VLAN 1 | 192.168.10.2 | 255.255.255.0 | 192.168.10.1 | Management |
| PC-Staff1 | FastEthernet0 | 192.168.10.11 | 255.255.255.0 | 192.168.10.1 | Staff PC |
| PC-Staff2 | FastEthernet0 | 192.168.10.12 | 255.255.255.0 | 192.168.10.1 | Staff PC |
| PC-Student1 | FastEthernet0 | 192.168.10.21 | 255.255.255.0 | 192.168.10.1 | Student PC |
| PC-Student2 | FastEthernet0 | 192.168.10.22 | 255.255.255.0 | 192.168.10.1 | Student PC |
| Server-1 | FastEthernet0 | 192.168.10.50 | 255.255.255.0 | 192.168.10.1 | Optional server |

# 4. Router and Switch Configuration

Router R1:
enable
configure terminal
hostname R1
interface gigabitEthernet 0/0
ip address 192.168.10.1 255.255.255.0
no shutdown
end
write memory

Switch S1:
enable
configure terminal
hostname S1
interface vlan 1
ip address 192.168.10.2 255.255.255.0
no shutdown
exit
ip default-gateway 192.168.10.1
end
write memory

# 5. Connectivity Tests

PC-Staff1: ping 192.168.10.1 — expected reply from router.

PC-Staff1: ping 192.168.10.12 — expected reply from PC-Staff2.

PC-Student1: ping 192.168.10.22 — expected reply from PC-Student2.

PC-Staff1: ping 192.168.10.50 — expected reply if the server is connected.

PC-Staff2: ping 192.168.10.2 — expected reply from switch management interface.

If a test fails, check cables, interface status, IP addresses, subnet masks, gateways, and whether interfaces are enabled.

# 6. Basic Segmentation Design

| Group | VLAN | Subnet | Gateway | Access Idea |
| --- | --- | --- | --- | --- |
| Staff | 10 | 192.168.10.0/24 | 192.168.10.1 | Approved university systems |
| Students | 20 | 192.168.20.0/24 | 192.168.20.1 | Learning resources; restrict staff systems |
| Servers | 30 | 192.168.30.0/24 | 192.168.30.1 | Only required services permitted |

A router or Layer 3 switch routes between VLANs. ACLs should control communication. Students may access learning resources but should not directly access confidential staff administration systems.

# 7. Student Questions — Model Answers

6. What device connects different IP networks?

A router connects different IP networks and forwards packets between them. A Layer 3 switch can also route between IP networks or VLANs. In this lab, R1 acts as the default gateway.

7. Why is segmentation useful?

Segmentation divides a network into smaller sections. It limits unnecessary communication, reduces malware spread, protects sensitive systems, improves monitoring, and permits different security rules for staff, students, and servers.

# 8. Deliverable Checklist

Network diagram

Configuration table

Router and switch configuration

Connectivity test results

Staff/student segmentation idea

Answers to both student questions

# 9. Conclusion — Model

The network uses one router, one switch, four PCs, and an optional server. Devices receive addresses from 192.168.10.0/24, and the router provides the default gateway. Ping tests verify connectivity. A more secure design separates staff, students, and servers into VLANs and subnets with routing and ACL rules controlling access.